Practical no. 1: Preparing the IoT Hardware

- Set up Raspberry Pi OS / Arduino IDE

- Configure GPIO settings and test basic connectivity

- Demonstrate pin layout and onboard peripherals

- Set up Raspberry Pi OS / Arduino IDE

### Part A – Setting Up Arduino (Using Tinkercad or Hardware)

#### In Tinkercad (Virtual Setup)

- Open Tinkercad Circuits → Log in or sign up.

- Click Create New Circuit.

- Drag and drop:

  - Arduino Uno board

  - LED

  - Resistor (220Ω)

- Connect:

  - LED anode (+) → Pin 13

  - LED cathode (–) → GND

- Click Code → Text and write:

void setup() {

pinMode(13, OUTPUT);

}

void loop() {

digitalWrite(13, HIGH);

delay(1000);

digitalWrite(13, LOW);

delay(1000);

}

- Click Start Simulation.
The LED blinks every second, showing correct GPIO output control.

### Part B. Setting Up Raspberry Pi OS (Hardware Only)

- Download and install Raspberry Pi Imager on your computer.

- Insert microSD card → Open Raspberry Pi Imager.

- Select:

  - Operating System: Raspberry Pi OS (32-bit)

  - Storage: Your microSD card

- Configure:

  - Wi-Fi, SSH, and username/password

- Click WRITE → Wait until process completes.

- Insert SD card into Raspberry Pi → Connect keyboard, mouse, monitor, and power.

- Complete setup wizard on first boot.

Raspberry Pi is now ready with OS installed.

- Configure GPIO settings and test basic connectivity

- Connect an LED between:

- GPIO17 (Pin 11) → Resistor → LED → GND (Pin 6)

- Open terminal and enter:

gpio -g mode 17 out

gpio -g write 17 1   # Turn ON

gpio -g write 17 0   # Turn OFF

LED turns ON/OFF — confirming GPIO works.

- Demonstrate pin layout and onboard peripherals.

## 1. Arduino Uno Pin Layout and Peripherals

- Arduino Uno is a microcontroller board based on the ATmega328P.

- It has multiple digital and analog I/O pins, power pins, and built-in peripherals used to connect sensors, actuators, and other devices.

### Pin Categories:

| Pin Type | Pin Numbers | Description / Purpose |
| --- | --- | --- |
| Digital Pins | 0–13 | Used for digital input or output (HIGH/LOW). Example: controlling LEDs or reading switches. |
| Analog Pins | A0–A5 | Used for reading analog sensor values (e.g., temperature, light, potentiometer). Converts analog voltage to digital values using the ADC (10-bit). |
| PWM Pins | ~3, 5, 6, 9, 10, 11 | Provide Pulse Width Modulation output to control motor speed or LED brightness. |
| Power Pins | 3.3 V, 5 V, GND | Used to power sensors and other low-power components. |
| Reset Pin | RESET | Resets the microcontroller to restart the program. |
| Built-in LED | Pin 13 | Useful for testing; blinks during the Blink program. |

Onboard Peripherals:

| Peripheral | Description |
| --- | --- |
| USB Port | For uploading code and serial communication with a computer. |
| Power Jack | Provides external 7–12 V supply if USB is not used. |
| Crystal Oscillator (16 MHz) | Keeps the microcontroller’s clock timing accurate. |
| Voltage Regulator | Converts input voltage to stable 5 V/3.3 V. |
| ATmega16U2 Chip | Acts as a USB-to-serial converter for communication. |

## 2. Raspberry Pi Pin Layout and Peripherals

- Raspberry Pi is a single-board computer with a 40-pin GPIO header.

- It supports digital I/O, communication protocols (I²C, SPI, UART), and includes various built-in hardware interfaces.

### GPIO Header Layout

The Raspberry Pi GPIO header has 40 pins, including:

- 26 GPIO pins (programmable for input/output)

- Power pins: 3.3 V, 5 V

- Ground pins (GND)

- Special-purpose pins for communication protocols

| Type | Example Pins | Purpose |
| --- | --- | --- |
| Power Pins | 3.3 V (Pin 1, 17), 5 V (Pin 2, 4) | Power external sensors and circuits. |
| Ground Pins | Multiple (e.g., Pin 6, 9, 14) | Common ground connection. |
| GPIO Pins | GPIO 2 – GPIO 27 | General-purpose digital I/O for LEDs, switches, etc. |
| UART (Serial) | GPIO 14 (TX), GPIO 15 (RX) | For serial communication with microcontrollers. |
| I²C | GPIO 2 (SDA), GPIO 3 (SCL) | For sensor communication (e.g., accelerometer, RTC). |
| SPI | GPIO 10 (MOSI), GPIO 9 (MISO), GPIO 11 (SCLK) | For high-speed data exchange with sensors/displays. |

Onboard Peripherals:

| Peripheral | Description |
| --- | --- |
| HDMI Port | Connects to display or monitor. |
| USB Ports | Connects keyboard, mouse, or storage devices. |
| Ethernet Port | Provides wired internet connectivity. |
| Audio Jack | Output for speakers or headphones. |
| Wi-Fi & Bluetooth | Wireless connectivity for IoT applications. |
| Camera Interface (CSI) | Connects Pi Camera for image/video processing. |
| Display Interface (DSI) | Connects Raspberry Pi Touch Display. |
| MicroSD Slot | Holds operating system and storage. |