Practical 2: GPIO – Light the LED (with and without Button)

- Blink LED using Python (Raspberry Pi) or C++ (Arduino)

- Add a push button to toggle LED ON/OFF

### Light an LED (without Button)

Wiring:

- Connect the LED’s anode (+) to a digital pin (e.g., pin 13)

- Connect the LED’s cathode (-) to a resistor (220Ω–330Ω) → GND

Code:

int ledPin = 13;  // Pin connected to LED

void setup() {

pinMode(ledPin, OUTPUT);  // Set LED pin as output

}

void loop() {

digitalWrite(ledPin, HIGH);  // Turn LED on

// LED stays ON permanently

}

### Blink LED

Code:

int ledPin = 13;  // Pin connected to LED

void setup() {

pinMode(ledPin, OUTPUT);

}

void loop() {

digitalWrite(ledPin, HIGH);  // Turn LED on

delay(500);                  // Wait 500ms

digitalWrite(ledPin, LOW);   // Turn LED off

delay(500);                  // Wait 500ms

}

This will blink the LED every 1 second (0.5s ON + 0.5s OFF).

### Toggle LED with Push Button

Wiring:

- LED: Same as before

- Push button:

  - One leg → digital pin (e.g., pin 2)

  - Other leg → GND

  - Use internal pull-up to simplify wiring

Code:

int ledPin = 13;

int buttonPin = 2;

bool ledState = false;      // Track LED state

bool lastButtonState = HIGH;

void setup() {

pinMode(ledPin, OUTPUT);

pinMode(buttonPin, INPUT_PULLUP);  // Enable internal pull-up resistor

}

void loop() {

bool buttonState = digitalRead(buttonPin);

// Check for button press (transition from HIGH to LOW)

if (lastButtonState == HIGH && buttonState == LOW) {

ledState = !ledState;             // Toggle LED state

digitalWrite(ledPin, ledState);   // Apply new state

delay(50);                         // Simple debounce

}

lastButtonState = buttonState;

}

LED will blink when you click on  button.