Practical no. 4:

8x8 LED Grid Control (Matrix LED Programming)

- Connect an 8×8 LED matrix module  Program animations or scrolling text patterns

- Explore logical formulas for patterns

### Components

- Arduino UNO

- LEDs

- Resistors (220Ω)

Connections:

### ROW Pins (Anodes → Arduino)

| Row | Arduino Pin |
| --- | --- |
| R0 | 2 |
| R1 | 3 |
| R2 | 4 |
| R3 | 5 |
| R4 | 6 |
| R5 | 7 |
| R6 | 8 |
| R7 | 9 |

### COLUMN Pins (Cathodes → via Resistors → Arduino)

| Column | Arduino Pin |
| --- | --- |
| C0 | 10 |
| C1 | 11 |
| C2 | 12 |
| C3 | 13 |
| C4 | A0 |
| C5 | A1 |
| C6 | A2 |
| C7 | A3 |

- Each column must have one resistor (220Ω)

Code:

int rowPins[8] = {2,3,4,5,6,7,8,9};

int colPins[8] = {10,11,12,13,A0,A1,A2,A3};

void setup() {

for(int i = 0; i < 8; i++) {

pinMode(rowPins[i], OUTPUT);

pinMode(colPins[i], OUTPUT);

}

}

void loop() {

showDiagonal();   // display diagonal pattern

}

// Display diagonal LEDs

void showDiagonal() {

for(int row = 0; row < 8; row++) {

digitalWrite(rowPins[row], HIGH);  // activate row

for(int col = 0; col < 8; col++) {

if(row == col) {

digitalWrite(colPins[col], LOW);   // LED ON

} else {

digitalWrite(colPins[col], HIGH);  // LED OFF

}

}

delay(2);   // small delay for multiplexing

digitalWrite(rowPins[row], LOW);  // deactivate row

}

}

Output: