Practical 8:

Web Trigger – Control GPIO from Web Server

- Host a simple Flask web app (Raspberry Pi) or ESP Web Page (Arduino)

- Control a set of LEDs via buttons on a webpage

- Ensure real-time response and feedback

### LED Connections

| LED | Arduino Pin | Series Resistor | GND |
| --- | --- | --- | --- |
| RED LED | Pin 8 | 220Ω | GND |
| YELLOW LED | Pin 9 | 220Ω | GND |
| GREEN LED | Pin 10 | 220Ω | GND |

### Wiring instructions

- Place three LEDs on the breadboard

- Connect each LED’s cathode (-) to GND (use the blue rail)

- Connect each anode (+) to:

  - Resistor → Arduino pin

- Pins to use:

  - Pin 8 → RED

  - Pin 9 → YELLOW

  - Pin 10 → GREEN

Write following code:

int redLED = 8;

int yellowLED = 9;

int greenLED = 10;

void setup() {

pinMode(redLED, OUTPUT);

pinMode(yellowLED, OUTPUT);

pinMode(greenLED, OUTPUT);

Serial.begin(9600);

Serial.println("Send r/y/g to toggle LEDs");

}

void loop() {

if (Serial.available()) {

char c = Serial.read();

if (c == 'r') digitalWrite(redLED, !digitalRead(redLED));

if (c == 'y') digitalWrite(yellowLED, !digitalRead(yellowLED));

if (c == 'g') digitalWrite(greenLED, !digitalRead(greenLED));

}

}

Output:

 When you run the simulation, all LEDs start OFF.

 You type a letter in the Serial Monitor:

- r → Red LED turns ON (or OFF if it was already ON)

- y → Yellow LED turns ON/OFF

- g → Green LED turns ON/OFF

 Each LED toggles every time you type its letter.

 So it simulates controlling LEDs from a web page, but using the Serial Monitor.